LAN control sample software for U3700  (V14)

V 1.4

ADVANTEST

- Install sample software into PC

You might see some strange characters, but don’t mind.  (Japanese Based…)

- Set U3700 IP address

System – Remote Control – LAN IP address – Network Setting

Fif.1 shows Point-to-Point setting example.

If you want to connect to LAN network, please ask IP address to network manager.

You can use DHCP: check the Obtain an IP address automatically, and push Apply.

You can read this Analyzer’s IP address.

- Set PC IP address

Just only for Point-to-Point connection. No necessary for LAN network connection.

For WIN2000

My network – Right click - Properties –

Local Area Connection - Right click - Properties –

Internet Protocol (TCP/IP) – Properties

Set IP Address

This IP address has to be set difference number from U3700

- Connect LAN cable

Have to use CLOSS CABLE for Point-to-Point connection

- Open sample software

Start – Programs – U3700 LAN Client (V14) - – LAN RMT TraceIQ (V14)

Set U3700 IP address

The Port No is 5025

- Push Connect

When the connection has connected, the message will be open as follow.

If you check the Monitor, the communication log will be displayed.

Write GP-IB command at Sending Text and push “Send Text” key

And then you can control U3700 series.

Send query command and then the returned data will be displayed.

- Push “Waveform View” key, the measurement screen will be displayed.

-There are basic parameter setting key and can change parameters.

(Center, Span, Ref, RBW, VBW, Sweep Time…)

-The “PRESET” key preset U3751 setting.

-The “CLOSE” key goes back to main menu.

-The “BMP” key saves Bit Map file into PC.

Example: Bit Map File

- Getting IQ data  (using option53/54/55/56)

If U3700 has the Time Domain option, you can get IQ data.  * IQ data = Real (Single)

IQ getting button

IQ data number

Display step number

- Shut down the program

First, back to Main Menu.

The “Close” key stops the LAN connection.

The “EXIT” key shuts down this program.