LAN control sample software for U3700 (V22)

ADVANTEST

- Install sample software into PC

You might see some strange characters, but don’t mind.  (Japanese Based…)

- Set U3700 series IP address

System – Remote Control – LAN IP address – Network Setting

Fif.1 shows Point-to-Point setting example.

If you want to connect to LAN network, please ask IP address to network manager.

You can use DHCP: check the Obtain an IP address automatically, and push Apply.

You can read this Analyzer’s IP address.

- Set PC IP address

Just only for Point-to-Point connection. No necessary for LAN network connection.

For WIN2000

My network – Right click - Properties –

Local Area Connection - Right click - Properties –

Internet Protocol (TCP/IP) – Properties

Set IP Address

This IP address has to be set difference number from U3700 series.

- Connect LAN cable

Have to use CLOSS CABLE for Point-to-Point connection.

- Open sample software  (Main Menu)

Start – Programs – U3700 LAN Client (V22) - – LAN RMT InvertDsp (V22)

Set U3700 series IP address	(Note: Auto Save/Power on recall items)

(Note: Please select A or B or C)

The Port No is 5025

- Push Connect

When the connection has connected, the message will be open as follow.

If you check the Monitor, the communication log will be displayed.

Write GP-IB command at Sending Text and push “Send Text” key

And then you can control U3700 series.

Send query command and then the returned data will be displayed.

Push “Waveform View” key, the measurement screen will be displayed.

There are basic parameter setting key and can change parameters.

(Center, Span, Ref level, RBW, VBW, Marker, Trace A/B Mode, Sweep Mode)

The “BMP” key saves Bit Map file into PC.

The “PRESET” key preset U3700 series setting.

The “CLOSE” key goes back to main menu.

Bit Map sample data.

- Shut down the program.

First, back to Main Menu.

The “Close” key stops the LAN connection.

The “EXIT” key shuts down this program.

Version 2.1 changing points:

- Marker key’s addition

- RF display = Marker Frequency + Offset Frequency

Version 2.2 changing points:

- Invert Display Key’s addition.

- You can see invert display trace A or B, when you check the menu A or B.