Waveform data acquisition (GPIB) sample software V 2.0

Advantest

- The waveform data acquisition sample software is started with Visual Basic.

- Trace DataOut Source/ScreenDsp.vpp(VB Project) of the sample software is loaded.

- Various data acquisition is programmed and you can refer to the source file, please.

(Trace: Bin, ASCII, Real Data)  (Time Domain IQ Real Data)

- The GPIB address of SA (Spectrum Analyzer) U3700 series is set “8”.

- SYSTEM - Remote Control - GPIB Address 8, from panel key.

- In initialization, it is set to 8. It changes if necessary.

(Photograph below is GPIB Address 8.)

- The GPIB cable is connected.

- Spectrum Analyzer is connected with the personal computer with the GPIB cable.

(The GPIB card is specified NI)

- The sample software is started.

- It starts with Visual Basic or ScreenDsp.exe is started directly.

- Spectrum Analyzer is controlled.

- The GPIB command is written in the frame of COMMAMD, and the “Set" button is pushed.

However, it is only a set command. (The query command: refer to chapter-6)

(example :  Center Frequency 20MHz:  CF 20MZ)

- The parameter data of Spectrum Analyzer is acquired.

- The GPIB command is written in the frame of COMMAMD, and the “Query" button

is pushed.  However, it is only a query command.

(example :  Reading of Center Frequency:  CF?)

- When the Waveform button is pushed, the waveform is seen.

(The key changes into Trace On/Off.)

- The button of Waveform changes into Trace On/Off since the second times.

- When the Copy button is pushed, waveform data is displayed in a left green frame

by the point value. At the same time, it is possible to copy it onto Excel etc,

because it is copied onto the clip board of Windows.

- Clear is a deletion of the screen and the point data.

- The transfer form is switched to Binary or ASCII by the Format menu.

(The amount of transfer form and transfer time is changed.)

- Waveform data is output with the Get Trace Real button by the floating point of 32bits.

1) The display area is a place of I:Data. (1001 points are seen in the scroll.)

2) L00 or more or option 2ch is necessary for the firmware version.

3) When a waveform display operates, this function is not possible.

Moreover, the level unit is dB mode.

- When the Time Domain analysis option (OPT53/54 etc.) is installed,

the IQ data can be output with the Get IQ-Data button.

1) The IQ data is separated, and the data is displayed in I area and Q area.

2) When a waveform display operates, this function is not possible.

3) When the display line exceeds 10k lines, it becomes 10k lines or less.

If you want the manual step size, you can write it to data box.

- End

- EXIT is pushed, and the program is terminated.

Reference: Format and program of waveform data

Example 1: Waveform data is read by ASCII format. (example of this sample software)

Private Sub readdat_ASCII()

Call ibdev(0, 8, 0, T10s, 1, 0, spa%)

Call ibconfig(spa%, 19, 1)

Call ibwrt(spa%, "DLIM0") 'U3751

Call ibwrt(spa%, "TS")

Call ibwrt(spa%, "TAA?")

For i = 0 To 1000

Call ibrd(spa%, trc(i))

Next i

Call ibwrt(spa%, "DLIM0") 'U3751

'Call ibwrt(spa%, "CONTS") 'Cont Sweep

'Call ibonl(spa%, 0) 'Reset GPIB

End Sub

Example 2: Waveform data is read by the BINARY format. (example of this sample software)

Private Sub read_data_bin()

Dim tr(1000) As Integer

Dim Rslt As Integer, tmp As Integer, i As Integer

Call ibdev(0, 8, 0, T10s, 1, 0, spa%) ' Set GPIB

Call ibconfig(spa%, 19, 0) ' Set EOI

Call ibwrt(spa%, "DLIM2") ' Set EOI @U3751

Call ibwrt(spa%, "TS") ' One Sweep

Call ibwrt(spa%, "TBA?") ' Set BIN mode

Call ibrdi(spa%, tr(), 1001 * 2) ' Read BIN data 1001 points

For i = 0 To 1000 Step 1

tmp = tr(i)

Rslt = (tmp And &HFF&) * 256

Rslt = Rslt + ((tmp And &HFF00&) / 256)

trc(i) = Str(Rslt) ' to Buffer 1001points

Next i

Call ibwrt(spa%, "DLIM0") 'Reset EOI @U3751

Call ibconfig(spa%, 19, 1) 'Reset EOI

'Call ibwrt(spa%, "CONTS") 'Cont Sweep

'Call ibonl(spa%, 0) 'Reset GPIB

End Sub